HOW TO SPECIFY

Models 61DVC, 61DHC

STEEL CURVED SPIRAL DUCT SUPPLY GRILLES – DOUBLE DEFLECTION

SUGGESTED SPECIFICATION: Furnish and install Nailor Model (select one) 61DVC or 61DHC Double Deflection Curved Spiral Duct Supply Grilles of the type and size as shown on the plans and air distribution schedules. The grilles shall have a dual set of "teardrop" blades spaced on 3/4" (19) centers. The frame shall be corrosion-resistant steel and rolled to match the specified radius. The finish shall be AW Appliance White (optional finishes are available). (Optional) Damper/Extractor (DEX), constructed of heavy gauge corrosion-resistant steel and operable from the face of the grille, shall be provided with all units. The manufacturer shall provide published performance data for the grille, which shall be tested in accordance with ANSI/ASHRAE Standard 70 – 2023.

Models 61SVC, 61SHC

STEEL CURVED SPIRAL DUCT SUPPLY GRILLES – SINGLE DEFLECTION

SUGGESTED SPECIFICATION: Furnish and install Nailor Model (select one) 61SVC or 61SHC Single Deflection Curved Spiral Duct Supply Grilles of the type and size as shown on the plans and air distribution schedules. The grilles shall have a single set of extruded aluminum "teardrop" blades spaced on 3/4" (19) centers. The frame shall be corrosion-resistant steel and rolled to match the specified radius. The finish shall be AW Appliance White baked enamel (optional finishes are available). (Optional) Damper/Extractor (DEX), constructed of heavy gauge corrosion-resistant steel and operable from the face of the grille, shall be provided with all units. The manufacturer shall provide published performance data for the grille, which shall be tested in accordance with ANSI/ASHRAE Standard 70 – 2023.

Models 51DVC, 51DHC

ALUMINUM CURVED SPIRAL DUCT SUPPLY GRILLES – DOUBLE DEFLECTION

SUGGESTED SPECIFICATION: 51DVC, 51DHC Double Deflection Furnish and install Nailor Model (select one) 51DVC or 51DHC Double Deflection Curved Spiral Duct Supply Grilles of the type and size as shown on the plans and air distribution schedules. The grilles shall have a dual set of extruded aluminum "teardrop" blades spaced on 3/4" (19) centers. The frame shall be made from a single piece aluminum design that is rolled to match the specified radius. The finish shall be AW Appliance White (optional finishes are available). (Optional) Damper/Extractor (DEX), constructed of heavy gauge corrosion-resistant steel and operable from the face of the grille, shall be provided with all units. The manufacturer shall provide published performance data for the grille, which shall be tested in accordance with ANSI/ASHRAE Standard 70 – 2023.

Models 51SVC, 51SHC

ALUMINUM CURVED SPIRAL DUCT SUPPLY GRILLES – SINGLE DEFLECTION

SUGGESTED SPECIFICATION: Furnish and install Nailor Model (select one) 51SVC or 51SHC Single Deflection Curved Spiral Duct Supply Grilles of the type and size as shown on the plans and air distribution schedules. The grilles shall have a single set of extruded aluminum "teardrop" blades spaced on 3/4" (19) centers. The frame shall be made from a single piece aluminum design that is rolled to match the specified radius. The finish shall be AW Appliance White (optional finishes are available). (Optional) Damper/Extractor (DEX), constructed of heavy gauge corrosion-resistant steel and operable from the face of the grille, shall be provided with all units. The manufacturer shall provide published performance data for the grille, which shall be tested in accordance with ANSI/ASHRAE Standard 70 – 2023.

Models 61L50C, 61L75C, 61L10C

CURVED SPIRAL DUCT SUPPLY LINEAR SLOT GRILLES

SUGGESTED SPECIFICATION: Furnish and install Nailor Model (select one) 61L10C (1", [25] slot), 61L75C (3/4", [19] slot) or 61L50C (1/2", [13] slot) of the sizes and capacities as shown on the plans and air distribution schedules. The linear shall be supplied in 1 – 4 slots wide as specified. The frame shall be corrosion-resistant steel and rolled to match the specified radius. The pattern deflectors shall have an aerodynamic 'ice tong' shape that can be adjusted to regulate the volume and direction of the airflow. The pattern deflector finish shall be black. The frame is to be AW Appliance White (optional finishes are available). The manufacturer shall provide published performance data for the grille, which shall be tested in accordance with ANSI/ASHRAE Standard 70 – 2023.

Models 45DLC1, 45DLC2

CURVED SPIRAL DUCT SUPPLY DRUM LOUVERS

SUGGESTED SPECIFICATION: Furnish and install Nailor Model (select one) 45DLC1 or 45DLC2 Spiral Duct Drum Louvers of the types and sizes as shown on the plans and air distribution schedules. The louver is to be manufactured from extruded aluminum and have a cylindrical adjustable drum that rotates. The blades are to have a paddle like profile that pivot on nylon bushings. The frame shall be made from a single piece aluminum design that is rolled to match the specified radius. A perimeter gasket is to be included around the frame and a felt seal is to be around the drum. The finish shall be AW Appliance White (optional finishes are available). (Optional) An opposed blade damper, constructed of heavy gauge corrosion-resistant steel and operable from the face of the grille, shall be provided with all units. The manufacturer shall provide published performance data for the louver, which shall be tested in accordance with ANSI/ASHRAE Standard 70 – 2023.